Installation & Configuration Manual

A complete step-by-step standalone guide for provisioning bare-metal hardware, configuring enterprise hardware RAID storage, and deploying a secured operating system.

Phase 1: Physical Setup & Initial Boot Sequence

- Power On & Hardware Readiness

  - Secure the server in the server rack.

  - Connect the dual power cables to independent Power Distribution Units (PDUs) for power redundancy.

  - Ensure the keyboard, monitor (VGA/DisplayPort), and network interface card (NIC) cables are firmly connected.

  - Press the Power Button on the front control panel.

  - Allow the server to execute its POST (Power-On Self-Test) routine.

- Enter System Setup

  - During the boot process, monitor the blue Dell EMC splash screen.(May varies depends on servers)

  - Tap the F2 key as soon as it appears on the screen (you will see Entering System Setup

populate in the top-left corner).(May varies depends on servers)

  - Once loaded, you will be presented with the System Setup Main Menu displaying three primary options:

System BIOS

iDRAC Settings 	Device Settings

Phase 2: RAID Controller Configuration (Dell PERC H730P)

This section maps directly to the user interface flow of the PERC H730P configuration utility.

System Setup Main Menu

│

▼

Device Settings

│

▼

RAID Controller in Slot 4 (PERC H730P)

┌───────┴────────────────────────┐

▼	▼

Configuration Management	Virtual Disk Management (Clear, Select Disks, Create)	(Verify final RAID volumes)

Navigating to the RAID Controller Interface

  - From the System Setup Main Menu, click on Device Settings.

  - Select RAID Controller in Slot 4: Dell PERC Configuration Utility.(May varies depends on servers)

  - You will arrive at the controller's Main Menu showcasing: 	Configuration Management

Controller Management 	Virtual Disk Management

Physical Disk Management 	Hardware Components

- Purging Existing Configurations

CRITICAL WARNING: Cleaning configuration metadata permanently destroys any data currently mapped to the hard drives. Ensure any critical legacy data has been backed up before executing this step.

  - Navigate to Configuration Management.

  - Click on Clear Configuration.

  - Check the Confirm checkbox.

  - Select Yes.

  - Once the screen displays "The operation has been performed successfully," click OK.

- Verify Disk Health & Physical Availability

  - Navigate back to the Controller's Main Menu.

  - Click on Physical Disk Management.

  - Check the status of all your connected drives. For an empty clean setup, you should verify

7 × SAS HDD drives (listed from 00:01:00 through 00:01:06 ), each showing: 	Capacity: 2.182TB

Status: Ready

Sector Size: 512B

- Step-by-Step: Building the RAID Virtual Disks

To balance reliability and performance, we split our 7 physical disks into two separate virtual disks (RAID arrays):

Virtual Disk 0 (OS Partition): Constructed with RAID 1 (Mirrored) using 2 disks. Highly fault- tolerant for system stability.

Virtual Disk 1 (Data Volume): Constructed with RAID 5 (Distributed Parity) using the remaining 5 disks. Optimizes speed and raw disk capacity.

Step 4A: Configure Virtual Disk 0 ( OS )

  - Go to Main Menu → Configuration Management → Create Virtual Disk.

  - Set Select RAID Level to RAID1.

  - Click on the Select Physical Disks link. 	Under filter configurations, select:

Select Media Type: HDD

Select Interface Type: Both

Logical Sector Size: Both

Under CHOOSE UNCONFIGURED PHYSICAL DISKS, check the boxes for:

Physical Disk

Physical Disk

Click Apply Changes. Click OK on the success screen.

  - Now, configure the parameters of the virtual disk: 	Virtual Disk Name: OS

Virtual Disk Size: 2.182 (Will auto-calculate to maximum safe capacity)

Virtual Disk Size Unit: TB

Strip Element Size: 256 KB (PERC default optimized for modern filesystems) 	Read Policy: Read Ahead

Write Policy: Write Through (Safest writing protocol, protects logs/system config) 	Disk Cache: Default

Default Initialization: No

  - Scroll down to the bottom and click Create Virtual Disk.

  - Check Confirm, click Yes, and click OK to acknowledge the successful creation.

Step 4B: Configure Virtual Disk 1 ( Datastore )

- From the same menu, click Create Virtual Disk once more.

- Change Select RAID Level to RAID5.

- Click Select Physical Disks.

Under CHOOSE UNCONFIGURED PHYSICAL DISKS, check the remaining 5 boxes:

Physical Disk 00:01:02

Physical Disk 00:01:04

Physical Disk 00:01:05

Physical Disk 00:01:06

Click Apply Changes and then click OK.

- Configure the parameters of this secondary volume: 	Virtual Disk Name: Datastore

Virtual Disk Size: 8.730 (Calculated using 4 × 2.182 TB usable storage with 1 TB

reserved for parity storage blocks) 	Virtual Disk Size Unit: TB

Strip Element Size: 256 KB

Read Policy: Read Ahead

Write Policy: Write Through

- Scroll down to the bottom and click Create Virtual Disk.

- Check Confirm, click Yes, and click OK.

- Final Hardware Storage Verification

  - Navigate to Main Menu → Virtual Disk Management.

  - Verify that your screen displays exactly two ready virtual disk arrays:

Virtual Disk 0: OS, RAID1, 2.182TB, Ready, Background Initialization 2%

Virtual Disk 1: Datastore, RAID5, 8.730TB, Ready

  - Click Finish on the bottom right corner of the screens.

  - Under the prompt "Confirm Exit", click Yes to reboot the server chassis.

Phase 3: Creating the Bootable Installation Media

While the physical server reboots, prepare your installation media using an external PC workstation.

- Download Your Target Operating System (OS) ISO

Choose and download the clean LTS enterprise version of your target operating system: 	Linux (RHEL Base): Rocky Linux / AlmaLinux Minimal

Linux (Debian Base): Ubuntu Server LTS

Windows Ecosystem: Windows Server Standard Edition

- Write the ISO using Rufus

  - Insert a blank USB flash drive (minimum size 16 GB) into your workstation.

  - Download and run Rufus (utility software).

  - Under Device, select your USB flash drive.

  - Click Select, browse, and open your downloaded OS ISO file.

  - Apply these configuration properties:

Partition Scheme: GPT (Mandatory for modern UEFI boot modes) 	Target System: UEFI (non-CSM)

  - Click Start. Wait until the process finishes completely, then safely eject your USB device.

Phase 4: ESXI Installation